# New vs. Old Pipeline Test Plan

rev3

8/21/26

## Purpose for this test:

To find a solution for the cameras at LB and HyCO4 that have been experiencing IR Video Pipeline Failures. These failures include a frozen video pipeline and seek cameras not being able to enumerate. Frozen video pipeline can occur during boot and or during runtime.

## Goal:

Determine whether the new SDK pipeline (seekcamera-gstreamer) is better, the same, or worse than the old pipeline (rtsp-seek-server). And furthermore if none of the pipelines can solve the seek enumeration problem can the new one at least have fewer and or delay the seek enumeration hardware failure. Solve / Mitigate the video pipeline failures.

Side Note: I am not sure that either of the pipelines can stop the Seek camera from enumeration failure - This might be a Seek issue.

### Differences between the Old and New Pipeline:

### OLD Pipeline

  - created by KDK

  - uses the sdk 4.1 version

  - started via crontab job that run system_init.sh at boot

  - rtsp-seek-server.sh: launched once at boot in crontab. Its while loop runs usbreset + relaunches every time the binary exits. Gaps: doesn't catch a hang (binary frozen, not exited), and can't self-heal if the wrapper process itself dies.

appsrc name=app0 format=time ! timecodestamper ! normfilter ! tee name=tp

tp. ! queue ! rtpgstpay name=pay0

tp. ! queue ! multifilesink max-files=10 location="/usr/local/bin/camera/ramdisk/image%d"

Element: appsrc (name=app0, format=time)

Role: SDK-fed raw thermal frame source, with time-based timestamps

────────────────────────────────────────

Element: timecodestamper

Role: Adds precise timestamps to each buffer for synchronization

────────────────────────────────────────

Element: normfilter

Role: Custom normalization filter

────────────────────────────────────────

Element: tee (name=tp)

Role: Splits the stream into two parallel branches — RTP streaming and

file save. GStreamer-native fan-out.

────────────────────────────────────────

Element: queue ! rtpgstpay (name=pay0)  [Branch 1]

Role: Payloads the stream for RTP streaming (needs an external sink like

udpsink to actually transmit it)

────────────────────────────────────────

Element: queue ! multifilesink (max-files=10, location="image%d")  [Branch 2]

Role: Saves frames to disk as a rotating buffer of up to 10 files

NEW Pipeline

  - written by sophia and claude

  - uses the sdk 4.4 version

  - systemd service (seekcamera-gstreamer.service)

  - Has an internal watchdog: absorbs USB timeout errors as warnings and keeps running; only exits and restarts if image frames stop being produced.

appsrc ! queue ! calibration ! pgmencode ! syntempsink

Element: appsrc

Role: SDK-fed raw thermal frame source

────────────────────────────────────────

Element: queue

Role: Buffering/threading boundary between the SDK callback thread and

the rest of the pipeline

────────────────────────────────────────

Element: calibration

Role: Applies the per-camera calibration (slope/offset fromcamera_imager) — this is the custom element read once at pipeline start (gst_calibration_start())

────────────────────────────────────────

Element: pgmencode

Role: Encodes the calibrated frame into PGM format

────────────────────────────────────────

Element: syntempsink

Role: Custom sink — writes/serves the final output to SynTemp Vision on port 2105

## Test 0 Design:

Steady-state endurance. Leave each pipeline running normally, with no deliberate reboot/restart stress at all, to catch slow-building problems that a constantly-restarting stress loop would mask by resetting state every cycle.

- 4 units total so that was there is replication in the setup. more details below:

| Unit/Test# | Pipeline | Type | Logging Cycle |
| --- | --- | --- | --- |
| PT0.1 | old (rtsp-seek-server) | Type A | 5 min |
| PT0.1NEW | new (seekcamera-gstreamer) | Type B | 5 min |
| PT0.2 | old (rtsp-seek-server) | Type A | 5 min |
| PT0.2NEW | new (seekcamera-gstreamer) | Type B | 5 min |

- All units have the same Power input

  - PoE Splitter (5V 3.5A) because I have qty 4 of each.

  - STEAMO Model:GPOE208 Gigabit PoE Switch

  - The same ethernet cables

- All units are burned with the same baseline OS image

  - changes are that TYPE B has the new pipeline ( binaries and services newSDK ) the old pipeline is there but not active

### Instruments that are logged on a N cycle time for N amount of days

- Both pipelines:

  - Is the device reachable via ssh

  - Seek USB enumeration status (grep the kernel log and command that reads what devices are connected to the USB hub)

  - Network behavior: ethernet link UP/DOWN count (grep the kernal log)

  - CPU load / CPU Temp / CPU Throttling

  - kernal log snapshot that is replaced everytime it is updated so im not storing exxcessive amounts of data.

- New Pipeline:

  - Number of USB error warnings

  - Number of systemd resets on the pipeline

  - If calibration data loaded on pipeline start

- Old pipeline:

  - image.pgm health - Are the bytes valid and when the last write was ?

### Things I am looking for:

- Does it self-recover from USB errors the old pipeline can't?

- Does the old/new pipeline fail, what is causing the failure and how can that be eliminated ?

- Any difference in Seek enumeration failures between pipelines?

- Any other faults/bugs, in either pipeline

### Prediction:

- if the new pipeline is better, it should show fewer/delayed Seek enumeration failures.

### Test Succession Criteria:

- The new pipeline has no lost seek enumeration or at least less than the old pipeline

### Next Steps:

- Get test planned reviewed and Approved

- Build a website that displays the static information about each test device and the current status of test device during a test run.

## Future Test 1 Design:

Two stress conditions, each run on one old-pipeline unit and one new-pipeline unit in parallel:

| Unit | Pipeline | Stress action | Cycle |
| --- | --- | --- | --- |
| PT1 | old (rtsp-seek-server) | software reboot | 2 min |
| PT1NEW | new (seekcamera-gstreamer) | software reboot | 2 min |
| PT2 | old (rtsp-seek-server) | pipeline restart | 60 sec |
| PT2NEW | new (seekcamera-gstreamer) | pipeline restart | 60 sec |

- All units have the same Power input

  - (PoE Splitter 5V 3.5A OR 12V 2A) because I have qty 4 of each.

  - STEAMO Model:GPOE208 Gigabit PoE Switch

  - The same ethernet cables

- All units are burned with the same baseline OS image.

### Instruments that are logged on a N cycle time

- Both pipelines:

  - Is the device reachable via SSH

  - Seek USB enumeration status (grep the kernel log and command that reads what devices are connected to the USB hub)

  - Network behavior: ethernet link UP/DOWN count (grep the kernel log)

  - CPU load / CPU Temp / CPU Throttling

  - kernel log snapshot that is replaced every time it is updated so I'm not storing excessive amounts of data.

- New Pipeline:

  - Number of USB error warnings

  - Number of systemd resets on the pipeline

  - If calibration data loaded on pipeline start

- Old pipeline:

  - image.pgm health - Are the bytes valid and when the last write was?